Cold/Warm Storage & Lab Instructions

Storage only

This means that biohazardous material is only stored in this room. No manipulation of biohazardous material will be done in this room, which includes centrifuging, shaking, pipetting, inoculating etc. This location does not need to be listed on your permit, but it must be found as a storage location in your biohazard inventory file. Please refer to SOP-Biosafety-11-Storage of Biohazard Samples for storage tips.

All cold/warm storage rooms will follow these guidelines (listed on door sign):

- NO biohazardous material to be manipulated in this cold room (see note below)

- NO cardboard or paper

- NO liquid nitrogen or dry ice (asphyxiation risk)

- NO flammable products

- NO storage on floors

- Clearly label all stored items with the lab supervisor name, date & biohazard sticker (if required)

- Organize samples and reagents in sealed plastic/metal containers

- Clean spills immediately

- Regularly dispose of contaminated or unwanted material (such as petri plates, media, buffer etc.)

- Dispose of all trash outside of the cold room (helps prevent mold)

- Turn off lights & make sure the door is closed and sealed before leaving (prevents condensation)

- Report mold contamination immediately to your supervisor

Signage on cold/warm storage rooms

- Cold/Warm Room Guideline sign provided by EHS

- Hazard Door Sign for each PI provided by EHS (both found under Laboratory Signage on our website).

Cold Lab

These rooms are considered cold labs, as biohazardous material is manipulated here on an open bench. Only authorized users of biohazard permit holders are allowed to perform biohazard activities in these labs. Since this room must be listed on your biohazard permit as a lab, you must also post a hazard door sign. The labs contact information and biohazardous material will be clearly listed on a biohazard door sign, along with the cold lab signage.

All cold/warm labs will follow these guidelines (listed on door sign):

- Biohazardous material may be manipulated in this cold room by authorized users of biohazard permits only (see note below)

- NO cardboard or paper

- NO liquid nitrogen or dry ice (asphyxiation risk)

- NO flammable products

- NO storage on floors

- Clearly label all stored items with the lab supervisor name, date & biohazard sticker (if required)

- Organize samples and reagents in sealed plastic/metal containers

- Clean spills immediately

- Regularly dispose of contaminated or unwanted material (such as petri plates, media, buffer etc.)

- Dispose of all trash outside of the cold room (helps prevent mold)

- Turn off lights & make sure the door is closed and sealed before leaving (prevents condensation)

- Report mold contamination immediately to your supervisor

Signage on cold/warm labs:

- Cold/Warm Lab Guidelines provided by EHS

- Hazard Door Sign for each PI provided by EHS (both found under Laboratory Signage on our website).

Biosafety inspections:

- These labs will be inspected on an annual basis all together. Even though they are listed on individual biohazard permits, many are shared spaces. The report will be sent to all PI’s who work in the space, or the dept (if it is managed by them).